| Number | Verification Plan Of IP_TIMER Trần Hồ Khánh Tân Name block | Feature | Verification item | Test Method | Pass condition |
| --- | --- | --- | --- | --- | --- |
| 1 | APB | Address decode | - Valid address of all 8 register | -Apb_read to each valid address | - all valid address have been asserted correctly - Expected : rdata = default_value |
| 2 | APB | Address decode | - Reserved address | -Apb_readto random address | - Apb_read: return RAZ ( 32’h0000_0000). |
| 3 | APB | Transfer data( no wait state) | -Apb_read/write | -Write random value to register. - Read the register after write function | - Apb_write expected complete on first cycle. Data transfer correctly in 1 cycle. ( Pready = 1) |
| 4 | APB | -Error response | - No error handling ( Pslverr always = 0) | - Write random value to TCR ( address valid). - Write random value to 0x20 - reserved address ( invalid address) | - Both in two cases Pslverr = 0 |
| 5 | Register | -TCR timer_en | - timer_en bit in TCR register | - Write 1 and 0 to timer_en bit. After that, read back | - timer_en = 1. The TCR_reg = 32’h0000_0001, and the counter start count. - timer_en = 0. The TCR_reg = 32’h0000_0000 and the counter stop count. |
| 6 | Register | - TCR div_en | - div_en bit in TCR register | - Write 1 and 0 to div_en bit. After that, read back | - div_en = 1. The TCR_reg = 32’h0000_0010 and the counter start count based on system clock ( sys_clk). - div_en = 0. The TCR_reg = 32’h0000_0000 and the counter start count based on div_val signal. |
| 7 | Register | - TCR div_val | - div_val bit in TCR register | - Write valid value( 4’b0000 -> 4’b1000) and read back | - Write div_val=4’b000 and read back matches write valid value. |
| 8 | Register | - TCR div_val | - div_val bit in TCR register | - Write invalid value ( 4’b1001 -> 4’b1111) and read back | - The div_val value not change ( keep the previous value that written to div_val). |
| 9 | Register | - TCR reset check | - TCR value after reset | - sys_rst_n = 0->1. After that check the value of TCR register | - Value of TCR register : 32’h0000_0100 ( timer_en = 0, div_en = 0, div_val = 4’b0001) |
| 10 | Register | - TDR0/1 value check | - TDR0/1 | - Write ( TDR0 = 32’h0000_0005) to TDR and start counter at normal speed . After 3 clock check TDR0/1 | - After 3 clock, value of TDR0 = 32’h0000_0008 and TDR1 = 32’h0000_0000. |
| 11 | Register | - TDR0/1 reset check | - TDR0/1 | - sys_rst_n = 0->1. After that check the value of TDR0/1 register | - Value of TDR0/1 register : 32’h0000_0000 |
| 12 | Register | - TCMP0/1 value check | -TCMP0/1 | - Write ( TCMP0 = 32’h0000_0010 and TCMP 1 = 32’h0000_0000) and read back | - Value after read back . TCMP0 = 32’h0000_0010 and TCMP 1 = 32’h0000_0000 |
| 13 | Register | -TCMP0/1 reset check | - TCMP0/1 value after reset | - sys_rst_n = 0->1. After that check the value of TCMP0/1 register | - Value of TCMP0/1 after reset : 32’hFFFF_FFFF |
| 14 | Register | -TIER interrupt check | - TIER int_en | - Write 1 to int_en and read back the value of TIER register and verify the function of timer_interrupt | - The value of TIER register= 32’h0000_00001. -Interrupt is enabled. |
| 15 | Register | -TIER reset check | - TIER | -- sys_rst_n = 0->1. After that check the value of TIER register | - Value of TIER after reset : 32’h0000_0000. |
| 16 | Register | -TISR interrupt check | - TISR int_st | Write 1 clears int_st, write 0 has no effect | - Set int_st=1 (via match), write 1 to clear; write 0 to set bit |
| 17 | Register | - TISR reset check | - TISR | sys_rst_n = 0->1. After that check the value of TISR register | - Value after reset : 32’h0000_0000. |
| 18 | Register | - THCSR ( Standard level) | halt_req is plain R/W, no function | Write 1/0 to halt_req bit, read back | Write THCSR=32'h0000_0001 -> Read THCSR bit0=1; Write THCSR=32'h0000_0000 -> Read THCSR bit0=0 (timer/counter behavior unaffected in both cases) |
| 19 | Counter_control | Default_mode | Cnt_en | Set timer_en=1, div_en=0; observe cnt_en | - Counter start count based on sys_clk. - cnt_en asserted every cycle when timer_en=1 |
| 20 | Counter_control | Control mode (div_en=1, div_val=0) | cnt_en | Set div_en=1, div_val=0; observe cnt_en | timer_en=1, div_en=1, div_val=0: sample cnt_en every clk for 10 cycles -> cnt_en=1 on all 10 samples (same as default mode) |
| 21 | Counter_control | Control mode (div_en=1, div_val=4’b0000 --> 4’b1000 ) | cnt_en pulses once every 2^div_val cycles | Set div_en=1, div_val=1-->8; observe cnt_en period | div_val=2 (limit=3): sample cnt_en over 8 cycles -> pattern = 0,0,0,1,0,0,0,1 (pulse at cycle 4 and cycle 8); repeat check for div_val=1,3,4,5,6,7,8 with period=2^div_val |
| 22 | Counter_control | Timer_en | Timer_en | -Set timer_en = 0 | - Counter do not count and stop |
| 23 | Counter | 64 bit count up | counter | - Run counter with cnt_en = 1 | - After N cycles, cnt = N cycles |
| 24 | Counter | Overflow behavior | overflow | - Run counter at 64’hFFFF_FFFF | - TDR0 = 32’hFFFF_FFFF and TDR1 = 32’hFFFF_FFFF after 1 cycles cnt = 64’h0000_0000 and keep count |
| 25 | Counter | Interrupt behavior | Interrupt | - Set TCMP = cnt and int_en = 1 | - tim_int asserted and counter(cnt) keep count. |
| 26 | Interrupt | Clear | Interrupt | - Set the timer_interrupt condition and clear condition ( tisr_wr_sel & wdata[0] = 1) at the exact sys_clk. | - int_st = 0, clear command wins. |
| 27 | Interrupt | Interrupt assertion | Tim_int | - Set the TCMP = 5 and cnt = 0. Run 5 cycles. Int_en = 1. | - After 5 cycle, cnt = 5 at #clk5 , tim_int = 1. |
| 28 | Interrupt | Interrupt masked | Tim_int | - Set the TCMP = 5 and cnt = 0. Run 5 cycles. Int_en = 0. | - After 5 cycle, cnt = 5 at #clk5 , tim_int = 0. Remain unchanged. |
| 29 | Interrupt | Clear Interrupt | Tim_int | - Set the TCMP = 5 and cnt = 0. Run 5 cycles. Int_en = 1. - At #clk 6 , write 32’h0000_0001 to TISR | - After 5 cycle, cnt = tcmp - At #clk5 , tim_int = 1 - At #clk6, TISR_int_st = 1 which make tim_int go from 1 --> 0. |
| 30 | Interrupt | Interrupt | Tim_int | - Set the TCMP = 5 and cnt = 0. Run 5 cycles. Int_en = 1. - At #clk 6 ,not write 1 to TISR register | - After 5 cycle, cnt = tcmp - At #clk5 , tim_int = 1 - At #clk6, cnt keep count up and tim_int = 1, unchanged untill write 1 to TISR_int_st. |